ITEMS INSPECTED

FOR SINGLE STORY HOUSE “R-1”

(Wood framed, Vinyl Siding)

NOTE: All “R-1” Type residences must have final inspection permit before inhabited!

- Items that require inspection before they are covered.

CONCRETE

- Spread Footing Excavation: for width, depth and shape. . (“Absolute Min.”16” Wide, 18” to the bottom from grade).

- Reinforcing: Of footing for size, quantity, type and location. (“Absolute Min.” 2-#4 spaced 3” from bottom of trench).

Thickness: Of concrete for footing. . (“Absolute Min.” 10” thickness of concrete).

- Foundation: Type (Reinforced concrete or block)

If concrete; width, height, reinforcing, size, quantity, type and location.

If block; width, height, reinforcing, type and size.

Anchor bolts: size, embedment, location, and spacing. (“Absolute Min.” 1/2” Dia. bolts embedded 7”, spa. at 6’-0” o/c, starting 1’-0” from ea. corner and at ea. side of opening).

- Slab: Thickness (Min. 4”), presents of vapor barrier, reinforcing, dimensions between expansion joints and spacing of control joints.

FRAMING

Band boards: Must be true, level and of continuous material.

- Floor joists: Spacing, size, connection to sills and band boards. Bearing length         (1 1/2” on 	wood, 3” on concrete). (Bridging required if span is more than ten feet (10’-0”). (In place of bridging; tongue and grooved, glued and nailed subfloor is acceptable if of the proper thickness).

- Sub floor: Type of material, spacing, thickness and how it is fastened. (If steel center beam is used it must have top flange stops to prevent overturning).

- Wall framing:

Studs: Type of material (wood or metal), spacing, connection method.

Sill plates: Any lumber in contact with concrete must be treated.

Top plates:  Must be doubled if wood. (Offset 48”)

Trusses: Member size, span, spacing, hurricane straps, connections and span.

- Ceiling Joists: Member size, spacing, span length, connection, support.

Rafters: Member size, spacing, span length, hurricane straps, connection, support.

Boxing: Exterior sheathing type, location, thickness and how it is fastened.

Opening headers: Member size, spacing, span length, connection, support.

Roof rafters: Member size, spacing, span length, connection, support.

ITEMS INSPECTED

FOR SINGLE STORY HOUSE “R-1”

(Wood framed, Vinyl Siding)

Sheathing: Roof type, installation, connection, thickness and how it is fastened.

PLUMBING

- Rough in drains & vents: Layout, direction of run, slope, and material of construction, size, and pressure test.

- Rough in supply lines: Layout, direction of run, expansion risers, material of construction, size.

- Finished drains: Pressure test, material of construction, components.

- Finished supply: Pressure test, material of construction, components.

Fixtures: Type, size, location, connections.

ELECTRICAL

Meter: Intended location.

Service panel: Size, location. (Min. 100 AMP Service).

Mast: Size, location, material, components.

- Grounding: Size, location.

- Outlet boxes: Location, number per circuit, type of material, attachment.

- Switch boxes: Location, circuit, type of material, attachment.

- Wiring: Routing (Openings, conduit, ECT.)

- Circuits: Number of circuits, items served by each circuit, AMP load.

GFI’s: Location, number.

Arc Fault Interrupters: All bedroom circuits.

Smoke detectors: Number, location, type.

Exterior: Number of outlets. (Min. two outlets on exterior, one in front and one in rear).

Exterior lighting: Front entrance and rear entrance.

MECHANICAL

Heating source: Min. BTU required to maintain winter temperatures per code requirements.

- Ductwork: Material, size, location, routing, support.

Exhaust fan: (If required).

Water heater: Size, location, type (Gas or Electric), vent if gas, seismic strap.

Roof venting: Type and location.