ITEMS INSPECTED

FOR ACCESSORY BUILDING “R-1”

(Wood framed, Vinyl Siding)

- Items that require inspection before they are covered.

CONCRETE

- Spread Footing Excavation: for width, depth and shape. . (“Absolute Min.”12” Wide, 12” to the bottom from grade).

- Continuous pour:  for width, depth and shape. . (“Absolute Min.”12” Wide, 12” to the bottom from grade, requires additional reinforcing).

- Reinforcing: Of footing for size, quantity, type and location. (“Absolute Min.” 2-#4 spaced 3” from bottom of trench). (Cont. pour requires two # 4’s top & bottom for “pole-barn” type const.).

Thickness: Of concrete for footing. . (“Absolute Min.” 10” thickness of concrete).

- Foundation: Type (Reinforced concrete or block)

If concrete; width, height, reinforcing, size, quantity, type and location.

If block; width, height, reinforcing, type and size.

Anchor bolts: size, embedment, location, and spacing. (“Absolute Min.” 1/2” Dia. bolts embedded 7”, spaced at 6’-0” o/c, starting 1’-0” from ea. corner and at each side of each door opening).

- Slab: Thickness (Min. 4”), reinforcing, dimensions between expansion joints and spacing of control joints.

FRAMING

- Wall framing:

Studs: Type of material (wood or metal), spacing, connection method.

Sill plates: Any lumber in contact with concrete must be treated.

Top plates:  Must be doubled if wood. (Offset 48”)

Trusses: Member size, span, spacing, connections.

- Ceiling Joists: Member size, spacing, span length, connection, support.

Rafters: Member size, spacing, span length, connection, support.

Boxing: Exterior sheathing type, location, thickness and how it is fastened.

Opening headers: Member size, spacing, span length, connection, support.

Roof rafters: Member size, spacing, span length, connection, support.

Sheathing: Roof type, installation, connection, thickness and how it is fastened.

ITEMS INSPECTED

FOR ACCESSORY BUILDING “R-1”

(Wood framed, Vinyl Siding)

PLUMBING (If Included)

- Rough in drains & vents: Layout, direction of run, slope, and material of construction, size, and pressure test.

- Rough in supply lines: Layout, direction of run, expansion risers, material of construction, size.

- Finished drains: Pressure test, material of construction, components.

- Finished supply: Pressure test, material of construction, components.

Fixtures: Type, size, location, connections.

ELECTRICAL

Disconnect panel: Size, location.

Mast/Underground: Size, location, material, components.

- Grounding: Size, location.

- Outlet boxes: Location, number per circuit, type of material, attachment.

- Switch boxes: Location, circuit, type of material, attachment.

- Wiring: Routing, conductors, (Openings, conduit, ECT.)

- Circuits: Number of circuits, items served by each circuit, AMP load.

- Lighting: number of outlets, location.

GFI’s: Location, number.