By

Gerald V Barta

Senior Instrumentation Specialist

June, 2022

Instrument Specs and Index

INTEGRATED INSTRUMENT INDEX DATA LIST

## INTEGRATED INSTRUMENT INDEX DATA LIST

- Instrument Specification Forms Integrated Data

About 103 fields are automatically copied from the ISA TR20 style instrument specification Form to an Excel Instrument Index spreadsheet, whenever a specification document is saved.

| ## Property Title | ## Data Description |
| --- | --- |
| ## Document Identifications |  |
| ## Document number | ## Specification Document number (must be unique) |
| ## Form_rev | ## Specification form number plus revision |
| ## Latest revision | ## Document latest revision identification |
| Publish Date | ## Date of document publish |
| Status | ## Status of document issue |
| ## Spec Id attri1value | ## Specification Identifications section definable field name |
| ## Spec id attribute 1 value | ## Specification Identifications section definable field value |
| ## Subject | ## Specification form title (document Subject description) |
| ## Company | ## Responsible Organization company identification |
| ## Comments | ## Word/SharePoint document comment |
| ## Reference specification | ## External document that applies to device requirements |
| ## File Name | ## Document full path or file name |
| ## Administrative Identifications |  |
| ## Project number | ## Project number |
| Sub project no | ## Sub project number |
| Project | ## Project title |
| Enterprise | ## Enterprise identification |
| Site | ## Site name |
| Area | ## Area identification acronym |
| Unit | ## Unit identification acronym |
| Admin Def attribute 1 name | ## Administrative Identifications section definable field name |
| Admin Def attribute 1 value | ## Administrative Identifications section definable field value |
| ## Service Identifications |  |
| Keywords (Tag no/Functional ID) | ## Primary Tag no/Functional identification |
| Related equipment | ## Related equipment identification |
| Service | ## Service description |
| Application service | ## Service description using manufacturer’s terminology |
| P_ID_Reference_dwg_number | ## P&ID or Reference drawing number |
| Upstr Line_Nozzle_Number | ## Upstream line or nozzle number |
| Upstr line pipe spec | ## Upstream line pipe spec |
| Upstr line nom rating | ## Upstream line nominal rating |
| Upstr line conn type | ## Upstream line connection type |
| Upstr line termn style | Upstream line termination style |
| Upstr line material type | ## Upstream line material type |
| Primary construction material | ## Material of pressure containing shell component |
| ## Safety Classifications |  |
| Inline hazardous area cl | ## Inline or Local Hazardous Area Class |
| Inline hazardous Div_Zone | ## Inline or Local Hazardous Area Division or Zone |
| Inline hazardous gr | ## Inline or Local Hazardous Area Group |
| Inline T Code | ## Inline or Local Hazardous Area Temperature Code |
| Environmental area | ## Description of environmental conditions at site of device |
| Testing/Listing agency | ## Suitable 3rd party listing agency identification |
| Criticality classification | ## Criticality classification |
| Safety Integrity level | ## Safety Integrity Level |
| Safety category | ## Safety Category |
| Signal loss failure mode | ## Signal loss failure mode |
| Supply loss failure mode | ## Supply loss failure mode |
| Material name | ## Process Material name |
| GHS health hazard | ## International ratings of material Health Hazards |
| Compliance standard | ## Compliance standard identification |
| Type of protection | ## Type of Protection |
| ## Component Design Criteria |  |
| Design Inlet press max | ## Design Inlet pressure maximum value |
| Design inlet press units | ## Design Inlet pressure maximum value units |
| PC Max press at design temp | ## Performance Characteristics Max pressure at design temp |
| PC Max press at design temp units | ## Performance Characteristics Max pressure at design temp units |
| Design Inlet temp max | ## Design Inlet temperature maximum value |
| Design inlet temp units | ## Design Inlet temperature maximum value units |
| PC Max design temp | ## Performance Characteristics Max design temp |
| PC Max design temp units | ## Performance Characteristics Max design temp units |
| Inlet temp min cond | ## Inlet temperature minimum flow condition |
| Inlet temp max cond | ## Inlet temperature max flow condition |
| Inlet temperature units | ## Inlet temperature units |
| PC Min working temp | ## Performance Characteristics Min working temperature |
| PC Min working temp units | ## Performance Characteristics Min working temperature units |
| PC Max working temp | ## Performance Characteristics Max working temperature |
| PC Max working temp units | ## Performance Characteristics Max working temperature units |
| Minimum ambient temp | ## Minimum ambient working temperature |
| Minimum ambient temp units | ## Minimum ambient working temperature units |
| PC Min ambient working temp | ## Performance Characteristics Min ambient working temperature |
| PC Min ambient working temp units | ## Performance Characteristics Min ambient working temperature units |
| Maximum ambient temp | ## Maximum ambient working temperature |
| Maximum ambient temp units | ## Maximum ambient working temperature units |
| PC Max ambient working temp | ## Performance Characteristics Max ambient working temperature |
| PC Max ambient working temp units | ## Performance Characteristics Max ambient working temperature units |
| Operating Def attribute 1 name | ## Operating Parameters section definable field name |
| Operating Def attribute 1 value | ## Operating Parameters section definable field value |
| Signal power source | ## Signal power source required for the device |
| Digital communication std | ## Digital communication standard for device |
| ## Modeling Physical Data |  |
| Estimated weight | ## Modeling Data Estimated weight |
| Estimated weight units | ## Modeling Data Estimated weight units |
| Face-to-face dimension | ## Modeling Data Face-to-face dimension |
| Face-to-face dimension units | ## Modeling Data Face-to-face dimension units |
| Overall width | ## Modeling Data Overall width |
| Overall width units | ## Modeling Data Overall width units |
| Overall height | ## Modeling Data Overall height |
| Overall height units | ## Modeling Data Overall height units |
| Overall depth | ## Modeling Data Overall depth |
| Overall depth units | ## Modeling Data Overall depth units |
| Removal clearance | ## Modeling Data Removal clearance |
| Removal clearance units | ## Modeling Data Removal clearance units |
| Mfr reference dwg | ## Identification of manufacturers drawing showing dimensional data |
| ## Calibration and Test |  |
| Prim Tag no Input_Output | ## Tag number or functional identification of the primary input or output signal |
| Prim_CAL_Input_Output Desc | ## Primary Calibration Input-Output Description |
| Prim_CAL_Input_LRV | ## Primary Calibration Input LRV |
| Prim_CAL_Input_LRV units | ## Primary Calibration Input LRV units |
| Prim_CAL_Action | ## Primary Calibration Action |
| Prim_CAL_Output LRV | ## Primary Calibration Output LRV |
| Prim_CAL_Output LRV units | ## Primary Calibration Output LRV units |
| Prim_CAL_Output URV | ## Primary Calibration Output URV |
| Prim_CAL_Output URV units | ## Primary Calibration Output URV units |
| Test pressure Input URV | ## Test pressure value |
| Test pressure Input URV units | ## Test pressure units |
| ## Component Identifications |  |
| Component type 1 | ## Primary Component type name |
| Component Manufacturer 1 | ## Primary Component Manufacturer name |
| Component Model number 1 | ## Primary Component Model number |

Note: Any additional Content Control property titles can be added to the Excel file columns and subsequent document savings will copy such additional data to the modified Instrument Index table.

- Manual Data Entry Properties of Instrument Index Table

About 64 additional manual data entry properties can be managed in the Instrument Index Data table:

| ## Extended Index Property Title | ## Data Description |
| --- | --- |
| ## Location Properties |  |
| ## Relative instrument location | ## Relative location such as field, local panel, remote panel, I/O building, etc. |
| ## Building number | ## Location building number |
| ## Floor/Elevation | ## Location Floor/Elevation |
| ## Storage location | ## Storage Location identification |
| ## Additional Loop or Tag Properties |  |
| ## Component_Symbol | ## Component symbol |
| ## Loop name | ## Loop name identifying parent common to all members of the loop |
| ## Sequence order | ## Loop sequence order of individual member of a loop |
| ## Identification_Tag_Number | ## Identification Tag Number |
| ## Functional_Identification_Letters | ## Functional Identification Letters |
| ## Alternate tag ID | ## Alternate tag identification of a device such as assigned by package equipment manufacturer, electrical interface equipment number or of a renamed device |
| ## Interlock number | ## Interlock number |
| ## Requires power supply | ## Requires power independent of their signal wiring |
| ## Installation package number | ## Installation package number |
| ## Loop check package | ## Loop check package |
| ## Milestones and Status |  |
| ## P&ID status | ## Status of P&ID activity such as future work, work on hold, pending deletion or pending scope change |
| ## P&ID Checked By | ## Initials of individual who checked the P&ID and Index data for completeness and consistency |
| ## Instrument status | ## Status of device such as existing, new, spare, abandoned in place, to-be-removed, etc. |
| ## Process data owner | ## Organization responsible for providing process data such as Process, Mechanical, Vessel, Electrical or packaged equipment |
| ## Process data required | ## Date for required process data specifications |
| ## Process data status | ## Status of process data |
| ## Requisition required | ## Date for issuing specification requisition |
| ## Specification package status | ## Status of specification package |
| ## Required on site | ## Date for receiving device on site |
| ## Purchase order status | ## Status of purchase order |
| ## Cause_Effect diagram status | ## Cause & Effect diagram status |
| ## Logic diagram status | ## Logic diagram status |
| ## Junction box drawing status | ## Junction box drawing status |
| ## 3D model status | ## Status of 3D model activity |
| ## Location drawing status | ## Location drawing status |
| ## Loop diagram status | ## Status of loop diagram activity |
| ## Air Purge connection status | ## Air Purge connection status |
| ## Electrical Signal conn status | ## Electrical Signal connection status |
| ## Environmental protect status | ## Environmental protection status |
| ## Process connection status | ## Process connection status |
| ## Support Mounting status | ## Support Mounting status |
| Piping isometric status | Piping isometric status |
| ## Construction package status | ## Construction package status |
| ## Procurement Properties |  |
| ## Supply responsibility | ## Organization responsible to supply the device such as instrumentation. piping, electrical, packaged equipment, etc. |
| ## Requestion number | ## Requisition number |
| ## Purchase order number | ## Purchase order number |
| ## Specification package | ## Specification package ID |
| ## Expected delivery date | ## Date of expected delivery |
| ## Construction package date | ## Date of Construction package issue |
| ## Drawing References |  |
| ## 3D model drawing | ## 3D drawing number |
| ## Air/Purge connection | ## Air/Purge connection detail drawing |
| ## Cause and Effect diagram | ## Cause and Effect diagram |
| ## Control strategy diagram | ## Control strategy diagram |
| ## Electrical location drawing | ## Electrical location drawing |
| ## Electrical/Signal connection | ## Electrical/Signal connection detail drawing |
| ## Environmental protection | ## Environmental protection detail drawing |
| ## Instrument location drawing | ## Instrument location drawing |
| ## Junction box | ## Junction box detail drawing |
| ## Logic diagram | ## Logic diagram number |
| ## Loop diagram | ## Loop diagram drawing |
| ## Piping isometric | ## piping isometric drawing number |
| ## Process connection | ## Process connection detail drawing |
| ## Process data sheet | ## Process data sheet |
| ## Process Flow Diagram | ## Process Flow Diagram |
| ## Support_Mounting | ## Support/Mounting detail drawing |
| ## System Properties |  |
| ## Instrument system | ## Abbreviation for the digital system which the device signal is connected to, such as DCS, PLC, ANALYZER, ESD, etc. |
| ## Associated I/O type | ## Associated digital system I/O component such as AI. AO, DI. DO, HART®, etc. |
| ## Associated I/O location | ## Location of associated I/O component such as building number, cabinet/rack number, panel number, etc. |
| ## Turnover System | ## Data packaging identification for transfer to the owner |
| ## Commissioning system | ## Commissioning system |

Note: Any additional manual entry property titles can be added to the Excel file columns.

- Custom Views of Instrument Index Data Table

About 19 basic custom views of the Instrument Index Data are provided and can be added to and configured:

- Add Components UserForm

- Additional Loop or Tag Properties

- Administrative Identifications

- Calibration Data

- Compare Design Conditions

- Component Identifications

- Document Identifications

- Drawing References

- Instrument Index Report Testing

- Instrument Specification Form Data

- Manual Data Entry of Index

- Modeling Physical Data

- Normal View (All for design)

- Process Data Comparison

- Process Specification Form Operating Parameters

- Procurement Properties

- Safety Classifications

- System Properties