- Good flaggers are responsible for:

- Guiding traffic

- Providing for the safety of the crew

- Providing information to the public

- All of the above

- The youngest, most inexperienced crew member always makes the best choice for a flagger.

### True

## False

- Flagging and traffic control associated with work zones are governed by law in Pennsylvania.  The following are applicable references:

- Title 67, Chapter 212

- Manual on Uniform Traffic Control Devices (MUTCD) Part 6

- PennDOT Publication 213

- All of the above

- What are the two basic types of equipment used by flaggers to control traffic:

1. ____________________________________

2. ____________________________________

- A Stop/Slow paddle should be used when flagging in the center of an intersection.

#### True

- False

- To stop traffic with a red flag, the flagger should wave the flag up and down.

- True

- False

- To signal traffic to proceed flaggers should use:

- Flag

- Free hand

- It is permissible to talk on a cell phone as long as the flagger remains attentive to their duties.

- True

- False

- Workers should rest near the flagger’s position where they will be most visible, and be able to converse with the flagger.

- True

- False

- When two flaggers are being used for two-way operations, each flagger should stand in the middle of the travel lane or close to the center of the roadway to be visible when stopping traffic.

- True

## False

- In a stationary work zone, how much buffer space should be provided from the flagger to the first work vehicle? ___________ feet.

- It is permissible to wear an orange or fluorescent yellow-green T-shirt to flag, rather than a vest.

A.	True

B.	False

- As per Act 229 guidelines (Additional Traffic-Control Devices in Highway Work Zones), the flagger is part of the “Active Work Zone”

A.	True

B.	False

- During hours of daylight, flaggers are required to wear:

- Sneakers

- Work gloves

## Protective helmet

##### None of the above

- At night, the red wand may be used as a substitute for a Stop/Slow paddle or a red flag.

- True

- False

- Between work shifts when flaggers are not in place, Flagger Ahead signs should be left in place to get drivers accustomed to the work zone.

- True

- False

- How many channelizing devices (e.g., cones) are required in a taper in a stationary operation when traffic is controlled by one or more flaggers?

- maximum of six

- minimum of six

- minimum of ten

## only as many as needed to make taper clearly visible

- When used for directing traffic, flags shall be:

- Red

- Orange

- Orange with white stripe

- The following shall be considered when deciding whether to utilize a single flagger:

- Traffic volume

- Length of work area

- Sight distance to flagger

- All of the above

- It is permissible to perform flagging duties while sitting in your truck for additional safety as long as you are in communication with the other flagger.

- True

- False

FLAGGER EXAM

# NAME: 	_______________________________________________

# COMPANY: _______________________________________________

# ADDRESS:	_________________________________________________

_________________________________________________

DATE: 	___________________________________________________

INSTRUCTIONS:  Read each question carefully and choose the best answer.

# EXAM GRADE: _________________________________