MATERIAL SAFETY DATA SHEET               April 28, 2005                                          Page 1 of 2

_________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Insulstrip Jell

_________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Section I

Manufacturer

_________________________________________________________________________

Ambion Corporation

37 Naugatuck Drive

Naugatuck, CT 06770

(203)723-1437

FOR EMERGENCIES:

P-ChEM:  1-800-424-5571

_________________________________________________________________________

Section II

Hazardous Ingredients

_________________________________________________________________________

Dichloromethane

C.A.S. No. 75-09-2

Common Names:     Methylene Chloride

OSHA PEL:  25 ppm (8 hour TWA)

125 ppm short-term exposure

ACGIH TLV:  50 ppm

Wt. % . . . . . . . 73

Formic Acid

C.A.S. No. 64-18-6

Common Names:       Methanoic Acid,

Formylic Acid,  Hydrogen Carboxylic Acid

OSHA PEL:  5 ppm    ACGIH TLV:  5 ppm

Wt. % . . . . . . . 12

Phenol

C.A.S. No. 108-95-2

Common Names:  Carbolic Acid,              Monohydroxy Benzene,  Phenic Acid

OSHA PEL:  10 ppm (skin)

ACGIH TLV:  5 ppm (skin)

Wt. % . . . . . . .   7

Toluene

C.A.S. No. 108-88-3

Common Names:      Methyl Benzene

OSHA TWA:  200 ppm

ACGIH TLV:  100 ppm

Wt. % . . . . . . .   3

D.O.T. Information:

Paint Related Material

Corrosive 8, PG II, UN3066

Emergency Response Guide No. 153

NFPA Rating:  H-3, F-1, R-1, SH-Cor

_________________________________________________________________________

Section III

Physical/Chemical Properties

_________________________________________________________________________

Boiling Point 	104ºF (40ºC)

Vapor Pressure (mm. Hg)	  305

Vapor Density (Air = 1) 	 2.9

Solubility in Water:  Not Soluble, coagulates

when mixed with water

Specific Gravity (H2O = 1) 	1.25

Melting Point	 N/A

Evaporation Rate (n-BuOH = 1) 	3.6

Appearance:  Clear, viscous, pink liquid.

Odor:  Sharp, acrid

_________________________________________________________________________

Section IV

Fire and Explosion Data

_________________________________________________________________________

Flash Point . . . . . None when tested per

DOT

Flammable Limits: Lower:  12%,

Upper:  19%

Extinguishing Media:  H2O, dry chemical,

CO2

Special Fire Fighting Procedures:  Fire fighters should wear NIOSH/MSHA approved self contained breathing apparatus for possible exposure to hydrogen chloride, formic acid, phenol and possible traces of phosgene.

Unusual Fire and Explosion Hazards:  Vapors concentrated in poorly ventilated areas can be ignited upon contact with a spark, flame or high intensity heat source.  This can occur at concentrations between 12% and 19% by volume.   Decomposition or burning can produce toxic gases.

_________________________________________________________________________

Section V

Reactivity Data

_________________________________________________________________________

Stability:  Stable

Conditions to Avoid:  Contact with flame or hot surfaces may produce toxic gases (phosgene, hydrogen chloride, formic acid, phenol).

Incompatibility (Materials to Avoid):  Oxygen under pressure, finely powdered metals (Al, Mg, Zn, Ti, etc.), caustic soda or potash and oxidizing agents.

Hazardous Decomposition or Byproducts:

Methylene chloride, Phenol, Formic Acid, Toluene, Phosgene, Hydrogen Chloride and Carbon Monoxide.

Hazardous Polymerization:  Will not occur.

_________________________________________________________________________

Section VI

Health Hazard Data

_________________________________________________________________________

Possible Exposure Paths:

Inhalation:  Possible

Skin/Eyes:  Possible

Ingestion:  Unlikely

Health Hazards, Acute:

Inhalation:  Severely irritating to nasal    	passages, throat and lungs.

Skin Contact:  Can cause severe chemical 	burn.

Eye Contact:  Causes severe and immediate

chemical burn.  Damage may be 	irreversible.

Ingestion:  Causes severe irritation and 	damage to mouth, throat and stomach.

Health Hazards, Chronic:

This product contains Methylene chloride which is a possible human carcinogen based on tests with lab animals.  Methylene chloride has been identified as a suspected carcinogen by OSHA .  Methylene chloride has also been associated with chromosome alteration in lab animals and with mutagenicity in lab animals.  This product contains phenol which is not listed by NTP (1985), IRAC (1983) nor by OSHA as of 1/30/86.  Phenol, however, is considered carcinogenic and neoplastic by RTECS criteria - tumors of the skin following dermal exposure in mice.  This product contains toluene which has not been identified as a carcinogen or probable carcinogen by NTP, IRAC or OSHA.  Toluene, however, has been associated with chromosome effects in workers exposed to high concentrations.

Signs and Symptoms of Exposure:

Skin:  Causes stinging burns that may not be immediately felt on less sensitive skin areas.  Irritation is generally felt within a minute or two.

Eyes:  Causes immediate and severe burns.  Vapors are irritating.

Inhalation:  Irritating to nasal passages, throat and lungs.

Ingestion:  Severely irritating and damaging to mouth, throat and stomach.

Medical Conditions Generally Aggravated by Exposure:  Preexisting eye, skin, respiratory and heart conditions may be aggravated by exposure to this product.

+ FIRST AID +

Ingestion (Swallowing):  If conscious, have patient drink large quantities of water and get immediate medical attention.  Never give anything by mouth to someone who is unconscious.

Inhalation (Breathing):  Remove patient from contaminated area.  If breathing has stopped, give mouth to mouth resuscitation, then oxygen if needed.  Get immediate medical attention.

Skin Contact:  Remove contaminated clothing at once while washing effected area with plenty of soap and water.  If there is skin irritation get medical attention.

Eye Contact:  Wash eyes with plenty of water, making sure to wash under eyelids, for 15 minutes and get immediate medical attention.

_________________________________________________________________________

Section VII

Safe Handling and Use

_________________________________________________________________________

Steps to Be Taken if Material is Released or Spilled:  Take steps to prevent eye or skin contact or inhalation.  Place leaking containers in well ventilated area.  Contain spill to minimize contaminated area and facilitate salvage or disposal.  To clean up spill, scrape up or use absorbent material.  Avoid run off into storm sewers or courses leading outside.  Wash contaminated area with alkaline cleaner and water.  All clean up and disposal should be carried out in accordance with federal, state and local regulations.

Waste Disposal Method:  This product is a hazardous material and should be disposed by a federally approved hazardous waste disposal facility.  The preferred method is incineration.

Precautions to be Taken in Handling and Storing:  Do not expose to temperatures above 90ºF.  Do not store above 75ºF.  Keep away from heat, sparks and flame.  Keep containers tightly closed.  Use on a first-in, first-out basis.  Store in D.O.T.

MATERIAL SAFETY DATA SHEET                                                                     Page 2 of 2

_________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Insulstrip Jell

_________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

approved containers. Use with adequate ventilation.  Take steps to prevent skin, eye or clothing contact.  Do not breathe vapor.  Wash thoroughly with soap and water after handling.  Wash contaminated clothing thoroughly before reuse.  Discard contaminated shoes and other leather clothing.

_________________________________________________________________________

Section VIII

Control Measures

_________________________________________________________________________

Respiratory Protection:  Use NIOSH approved organic vapor/acid gas respirator or canister respirator within the use limitations of these devices when lacking sufficient local ventilation to control vapors to a safe level.  In all other situations use a self-contained breathing apparatus..

Ventilation:  Use local exhaust when appropriate to control worker exposure.  Normal room ventilation only is not normally recommended.

Protective Gloves:  For workers using small amounts of this product as is typically done when stripping insulation from wires, it is generally better to not use protective gloves.  Over a period of time the stripper can penetrate protective gloves and attack the skin without the worker being aware of it.  If skin contact occurs without gloves, the worker will feel a tingling sensation almost immediately and have time to wash before any damage is done.  When handling larger quantities (more than a gallon) gloves made from polyvinyl alcohol are recommended.  Never immerse hands in the stripper, whether wearing gloves or not.  If glove contact occurs, wash gloves before continuing.

Other Protective Clothing and Equipment:  For operations where spills or splashes can occur, use impervious body covering and boots.  A safety shower and eye wash should be available.

Work/Hygienic Practices:  This material is often removed from wires after stripping by the use of small wipers.  These wipers should discarded after each use to avoid skin contact.  Be sure wipers are disposed in an environmentally safe manner.

_________________________________________________________________________

Section IX

Regulatory Information

_________________________________________________________________________Ingredient:  Dichloromethane	73%

C.A.S. No.: 	75-09-2

Hazardous (per SARA Title III) 	 Yes

Reason:	 Acute and Chronic Health

Reporting Required: 	Yes - 1000+ lb.

TSCA Inventory 	 Yes

Ingredient:  Formic Acid	12%

C.A.S. No.:	64-18-6

Hazardous (per SARA Title III) 	Yes

Reason:	 Acute Health

Reporting Required:	Yes - 5000+ lb.

TSCA Inventory	 Yes

Ingredient:  Phenol 	7%

C.A.S. No.:	 08-95-2

Hazardous (per SARA Title III) 	Yes

Reason:	Acute and Chronic Health, Fire

Reporting Required:	 Yes - 100+ lb.

TSCA Inventory	Yes

Ingredient: Toluene  	3%

C.A.S. No.:	 108-88-3

Hazardous (per SARA Title III) 	 Yes

Reason:	 Acute and Chronic Health, Fire

Reporting Required:	No

TSCA Inventory 	Yes

Ingredient: Trade Secret 	 5%

C.A.S. No.:	N/A

Hazardous (per SARA Title III) 	No

Reason: 	N/A

Reporting Required: 	No

TSCA Inventory	Yes

_________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________