TECHNICAL INFORMATION DATA SHEET

BC-176

BURNISHING COMPOUND POWDER

FOR COMMERCIAL AND INDUSTRIAL USE ONLY

NOT FOR RESALE

Ambion BC-176 is a non-abrasive burnishing compound powder that is designed to give a bright, lustrous finish to brass, steel, copper and its alloys and zinc.

USE

Ambion BC-176 should be used at concentrations of  0.5 to 2 oz. per gallon of water.  While light oil on the work may not interfere with the burnishing action, best results are obtained when the work is precleaned.  Plated surfaces may be burnished effectively with this compound.   All types of mass finishing equipment may be used including tumbling barrels, vibratory machines, etc.

PACKAGING

Ambion BC-176 is supplied in 450 lb. fiber drums.  Smaller containers are also available on request.

HANDLING

Ambion BC-176 contains no alkali or other harsh materials.  However, like all industrial chemicals, it should be treated with respect.  Wash hands thoroughly after using.  In the event of eye contact, rinse eyes with plenty of water for 15 minutes.  If irritation persists get medical attention.  For more detailed information, refer to the Material Safety Data Sheet for this product.