Model: DK-SW1WCP

700TVL Vandalproof Face Detection surveillance camera dome

Intelligent image adjustment

1. High contrast scene detection, automatically WDR

2. Illumination Scene Detection automatically adjusts 3D/2D Noise Reduction

(HDNR) and Digital Slow Shutter

3. Face Detection brightness automatically adjusts the dynamic exposure compensation

4. Face tracking detection, automatically adjust the image to enlarge

5. Night Infrared Detection automatically adjusts the exposure intensity

To  (HDNR)

To  (WDR)

To   (Face zoom)

(Light inhibition)

High Fogging Ability

Video processing features

. High-definition images

. Optical octave WDR

. 3D/2D (HDNR)

. Digital Shutter

. Light Inhibition

. Face metering compensation

. Digital processing through the fog

. Face zoom

. Auto Backlight Compensation

. Intelligent IR exposure

. Color rolling Inhibition

. 256 times digital zoom

. Privacy Shield

. Face Detection

. Motion Detection

. Dynamic / static elimination of dead pixels (4096)

. Translucent multi-level menu

. RS485 remote-control

. Multilayer Auto (auto exposure, auto-gain, auto white balance

Automatic iris control, auto focus detection)

. Image settings (brightness, contrast, saturation, sharpness, hue)

| Model | DK-SW12CP |
| --- | --- |
| Image Sensor | AVS03A |
| TV Line | 700TVL |
| Intelligent function | OSD, 3D/2D DNR, WDR |
| Resolution | 1020H×596V(PAL); 1020H×508V(NTSC) |
| TV System | NTSC/PAL |
| Min. Illumination | IR off: 0.001 lux IR on: 0.0lux |
| S/N Ratio | >50dB (AGC OFF) |
| Sync System | Internal line synchronization |
| Gamma Correction | 0.3~1.0 adjustable (usually set as 0.45) |
| Video Output | 1.0Vp-p,75Ω, Negative |
| Scanning system | PAL: 525 Lines, 50Field/S; NTSC: 525 Lines, 60Field/S |
| Shutter Speed | PAL: 1/50~1/10,000s; NTSC: 1/60~1/10,000s adjustable |
| White Balance | Auto,F/A,U30,CWF,D50,D65,D75 adjustable |
| Backlight Com. | Auto BLC |
| Lens | 2.8-12mm |
| IR Leds/Distance | 66pcs IR LED / IR Range 30~40m |
| Power Supply | DC12V / 3A (±10%) |
| Operation Temp. | -10ºC~+50ºCRH95%Max |
| Storage Temp. | -20ºC~+60ºCRH95%Max |