### Instructions

- 1. Adjust the position of your 3” X 5” tool rest so the
    top surface is level and the edge facing the wheel
    is within 1/8” of the wheel surface.

- 2. Tighten the cam lever and adjustment handle
    securely!

- 3. File or sand the edge of the platform rest facing
    you until it is smooth. Slightly chamfering the top
    edge is advisable.

- 4. Remove the rubber cap to expose the
    diamond tip.

- 5. Loosen the nylon knurled ring nut slightly.

- 6. Turn the round adjustment knob counter-
    clockwise to retract the position of the diamond
    close to the body.

- 7. Without the grinder running: Place the body of the
   dresser onto the top surface of your tool rest and
   slide the Tru-N-Dress across the wheel and adjust
   the position of the diamond until it very slightly
   contacts the wheel.

- 8. Move the diamond away from the wheel.

- 9. Put on a dust mask and a face shield and turn the
    grinder on.

- 10. Slide the Tru-N-Dress across the wheel with a
      smooth traverse, back and forth about 6 times.

- 11. Advance the position of the diamond very
      slightly by turning the adjustment knob.

- 12. Repeat step 10 and 11 a few times. Turn the
      grinder off and inspect the wheel. When the
      entire surface has been trued, move the tool rest
      to the other wheel and repeat the procedure.

- Call 352-472-5035 if you have any questions!

- Remember: Good sharpening begins with properly dressed wheels!

Set the angle of your tool rest so it points toward the axle of the grinder or slightly downward.

Make sure the tool rest is within 1/8” of the grinding wheel.

wheel

When truing the wheel, make light cuts and traverse across the wheel 6 to 8 times between adjustments.

## Tool rest

Smooth and slightly chamfer this edge.

Tool rest